_________________________________JBI, Inc._______

Chemical Blending

Safety Data Sheet								   Page 1

Bug & Tar Remover

Revision Date: 8/28/2024

Section 1: Company and Product Information

Product

Product Name: Bug & Tar Remover

Product Class: Aliphutic Solvent (Xylene, VM&PNaptha)

Company Identification

JBI, Inc.

4715 Fraley Drive

Timmonsville, SC  29161

24 Hour Emergency Telephone: (800) -424-9300 Chemtrec

General Information: (843)-346-4996

Section 2: Hazards Identification

Classification of the substance or  mixture

According to Regulation (HCS) paragraph (d) of 1910.1200

GHS Classification:  Flammable liquid 2, Aspiration Toxicity 1, Eye and skin Irritant 2A

Signal Word: Danger

Label Elements / GHS Hazard Symbols

Symbols

GHS Hazard Statement(s):

H225 Highly flammable liquid and vapor

H304 May be fatal if swallowed and enters airways

H315 Causes skin irritation

H320 Causes eye irritation

H336 May cause drowsiness or dizziness

GHS Precautionary Statement(s):

P202 do not handle until all safety precautions have been read and understood.

P210 Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking.

P280 Wear protective gloves/ protective clothing / eye protection / face protection.

P301 + P310 IF SWALLOWED: IMMEDIATELY CALL A POISON CENTER AND DOCTOR.

P308+p313 If exposed or concerned: Get medical advice / attention.

P331 NO NOT INDUCE VOMITING

P370 +P378 In case of fire: See Section 5 for extinguishing media.

P501 Dispose of contents / container to appropriate waste disposal facility, in accordance with local, regional, national, and international regulations.

P102 Keep out of reach of children.

P261 Avoid breathing dust / fumes / gas/ mist / vapors / spray.

P264 Wash hands thoroughly after handling

P305 + P351+ P338 If in eyes: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do.

Continue rinsing.

P333 + P313 If skin irritation or rash occurs: Get medical advice / attention.

P337 + P313 If eye irritation persist: Get medical advice / attention

P342 + P311 If experiencing respiratory symptonm: Call a POISON CENTER / Doctor

P361 + P364 Take off immediately all contaminated clothing and wash it before reuse.

_________________________________JBI, Inc._______

Chemical Blending

Safety Data Sheet								    Page 2

Bug & Tar Remover

Signs and symptoms of exposure:

Eye:	Avoid all eye contact. Irritation may occur. Rinse cautiously with water for several minutes. Remove contact lenses if 	present and able to do so. Continue rinsing.

Skin/ Hair:	 May cause mild irritation. Symptoms include redness, itching, and burning. Repeated or prolonged skin contact can 	 	produce moderate irritation (dermatitis). Take off all contaminated clothing, rinse skin with water / shower.

Ingestion: 	 Do not induce vomiting. Immediately seek medical attention. Immediately call a POISON CENTER or doctor.

Inhalation: Remove person to fresh air and keep comfortable for breathing. Call a POISON CENTER or doctor / physician if you  	 	 experience burning of throat, esophagus, sinus burning , or feel generally unwell.

Medical Conditions Aggravated by Exposure: Dermatitis. Dry skin and poor respiratory system.

HMS Rating:  	Health -1   Fire -4    Reactivity - 0     Personal  Protection - C

0= Insignificant 	1 = Slight	2=Moderate  3=High   4=Extreme

Section 3: Composition / Information on Ingredients

Substances

Composition  Information - Main constituents

| Chemical Name | CAS Number | %Range | OSHA PEL | ACGIH TLV |
| --- | --- | --- | --- | --- |
| Xylene VM&P Naptha | 1330-20-7 64742-89-8 | 50% 50% | -- -- | 100 ppm 300 ppm |

Section 4: First Aid Measures

Eye Contact: 	Flush eyes with water for at least 15 minutes. Hold eyelids apart to ensure complete irrigation of the eye and 			eyelid tissue. If easily accomplished, check for and remove contact lenses.  Do not use eye ointment. Seek 			medical attention.

Skin Contact:	Remove contaminated shoes and clothing. Flush affected area with large amounts of soap and water. Do not 			use ointments. Seek medical attention if tissue appears damaged or if pain or irritation persists.

Ingestion:		Do not induce vomiting. If spontaneous vomiting is about to occur, place victim’s head below knees. Place on 		                 left side with head down. Never give anything by mouth to an unconscious person. Do not leave victim 			unattended. Seek medical immediately. Begin CPR if needed.

Inhalation:		Move to fresh air. If unconscious place in recovery position and medical advice. Consult a physician after 			significant  exposure.

General:		Move out of dangerous area. Call a POISON CENTER or doctor if exposed and experience above symptoms or 		feel unwell. Show this Safety Data Sheet to medical staff. Symptoms of poisoning may occur appear several 			hours later. Do not leave the victim unattended.

_________________________________JBI, Inc._______

Chemical Blending

Safety Data Sheet								   Page 3

Bug & Tar Remover

Section 5: Fire-Fighting Measures

NFPA Flammability Classification:  NFPA Class-IB flammable liquid.

Flash Point: Closed Cup:  135’F

Lower Flammable Limit:  NA

Upper Flammable Limit:  NA

Hazardous Combustion Products:  Carbon dioxide, carbon monoxide, smoke, fumes, organic compounds.

Special Properties: Flammable Liquid! Vapors are heavier than air. It’s vapor can cause a flash fire. A vapor and air mixture can    		             create an explosion hazard in confined spaces such as sewers. If container is not properly cooled, it can rupture in 	    	             the heat of a fire.

Extinguishing Media: Use dry chemicals, carbon dioxide, foam,  water fog or spray. Streams of water may be 			 		ineffective. High volume water jet is unsuitable as extinguishing media.

Protection of Fire Fighters:

Firefighters must use full bunker gear including NIOSH-approved positive pressure self-contained breathing apparatus 	(SCBA) to protect against potential hazardous combustion or decomposition products and oxygen deficiencies. Evacuate 	area and fight the fire from a maximum distance.

Section 6: Accidental Release Measures

Flammable Liquid! Release causes and immediate fire or explosion hazard. Evacuate all non-essential personnel from immediate area and establish a “regulated zone” with site control and security. Eliminate all ignition sources. All Equipment used when handling this material must be grounded. Prevent spilled material from runoff.  Absorb or cover with dry earth, sand, or other non-combustible material and transfer to appropriate waste containers. Comply with all applicable local, state, and federal laws and regulations.

Section 7: Handling and Storage

Handling:

Use only with adequate ventilation and Personal Protection Equipment (PPE). A spill or leak can cause an immediate fire or explosion hazard. Ground all equipment. Keep container closed and do not handle near potential ignition sources. Avoid contact with oxidizing agents. Do NOT breathe vapor. Avoid contact with eyes, skin, and clothing. Do NOT take internally. A static electrical charge can accumulate with agitation and can ignite accumulated vapors. Empty containers may contain material residues which can ignite with explosive force. Observe label precautions. Open drums carefully as contents may be under pressure. Provide sufficient air exchange and exhaust in work rooms. Do not smoke, eat, or drink around this product. Container hazardous when empty.

Storage:

Keep container tightly closed. Store in cool, dry, well-ventilated area. Store only in approved containers. Do not store with oxidizing agents. Do not store at elevated temperatures or in direct sunlight. Protect containers against physical damage. Storage area must meet OSHA requirements and applicable fire codes. Observe label precautions. Containers which are opened must be carefully resealed and kept in upright position to prevent leakage.

Section 8: Exposure Controls and Personal Protection

Engineering Controls: Use only with adequate ventilation. Avoid breathing vapor or mist. Local exhaust preferred.

Eye Protection: Safety glasses with side shields are recommended. Chemical goggles are preferred when chance misting, splashing, or spraying of this material. A suitable emergency eye wash water and safety shower should be located near the work station.

Skin Protection: Avoid skin contact. Use heavy duty chemical resistant gloves. Wear long-sleeved fire-retardant garments. If splashing or exists wear an apron, boots and additional facial protection. Wash hands with plenty of soap and water after handling.

Respirator Protection: If required, a properly fitted NIOS/MSHA approved respirator for hazardous ingredients found in SECTION 3.

_________________________________JBI, Inc._______

Chemical Blending

Safety Data Sheet								    Page 4

Bug & Tar Remover

Section 9: Physical and Chemical Properties

Boiling Point: 308 ‘ Degrees F

Vapor Pressure (mm Hg): <10 Hg

Vapor Density (AIR = 1): 5.0

Solubility in Water: non soluble

Specific Gravity (H2O = 1): 0.80

Melting Point: NA

Evaporation Rate: (Ether) <1.0

Appearance and Odor: Clear  with Solvent Odor / Citrus

Section 10: Stability and Reactivity

Chemical Stability: Stable

Conditions to Avoid: Keep away from heat, flame and other potential ignition sources including static charge.

Hazardous Polymerization: Will not occur

Materials Incompatibility: Strong acids, alkalies, and oxidizers such as liquid chlorine and oxygen.

Hazardous Decomposition Products: Carbon dioxide, carbon monoxide, smoke, fumes, and/or unburned hydrocarbons.

Section 11: Toxicology Information

Listed as IARC, NTP, OSHA Carcinogen:  None

Section 12: Ecological Information

Not available

Section 13: Disposal Considerations

Waste from this product is hazardous (D001 and D018) as defined by the Resource Conservation and Recovery Act (RCRA) 40 CFR 261. Waste must be tested for ignitability to determine the applicable EPA hazardous waste numbers. Dispose of in accordance with federal, state and local regulations. Do not dispose of in sewer.

Section 14: Transport Information

D.O. T. REQUIREMENTS 49CFR  172.101:  UN1268, Petroleum Products, N.O.S.,3, II, Flammable Liquids (class 85)

Section 15: Regulatory Information

RQ (Reportable Quantity) 49CFR 3172.101:

CERCLA Reportable Quantity

Components	CAS-No.		Component RQ (lbs)	Calculated product RQ (lbs)

Xylene		1330-20-7		100		12454.851165

VM&P Naptha	64742-89-8	none		none

_________________________________JBI, Inc._______

Chemical Blending

Safety Data Sheet								  Page 5

Bug & Tar Remover

SECTION 311/312 HAZARD CLASS - 40CFR 370.2

Immediate	   		(x)

Delayed 	                 		 (x)

Fire			(x)

Reactive			( )

Sudden Release of  Pressure	( x)

SARA 313 COMPONENTS - 40CFR 372.65:   None

WORKPLACE HAZARDOUS INFORMATION SYSTEM (WHMIS)

Ingredient				Max% by Wt.	LD50		LC50

Xylene				50%		Not Established          Not Established

VM& P Naptha			50%	          	Not Established	Not Established

CALIFORNIA PROPOSITION 65:  None

Section 16: Other Information

Although the information contained herein is believed to be reliable, it is furnished without warranty of any kind. This information is  not intended to be all-inclusive as to the manner and conditions of use, handling, and storage.

JBI urges each recipient of this Safety Data Sheet to study it carefully to become aware of and understand the hazards associated with the product.  Each recipient should inform all employees, customers, and all users of this product with information contained in this SDS.