_________________________________JBI, Inc._______

Chemical Blending

Safety Data Sheet								   Page 1

Yellow Monster

Revision Date: 8-28-2024

Section 1: Company and Product Information

Product

Product Name: Yellow Monster

Product Class: Water Based Degreaser

Company Identification

JBI, Inc.

4715 Fraley Drive

Timmonsville, SC  29161

24 Hour Emergency Telephone: (800) -424-9300 Chemtrec

General Information: (843)-346-4996

Section 2: Hazards Identification

Classification of the chemical in accordance with paragraph (d) of 1910.120:

GHS Classification:  Corrosive 1A

Label Elements / GHS Hazard Symbols

Symbols   Signal Word: Warning

GHS Hazard Statement(s):

H302 + H312 Harmful if swallowed or in contact with skin

H304 May be fatal if swallowed and enters airways

H314 Causes sever skin burns and eye damage

GHS Precautionary Statement(s):

P101 If medical advice is needed, have product container or label at hand.

P102 Keep out of reach of children.

P103 Read label before use.

P202 Do not handle until all safety precautions have been read and understood.

P261 Avoid breathing dust/ fumes/ gas/ mist/ vapors/ spray.

P264 Wash hands thoroughly after handling.

P280 Wear protective gloves / protective clothing / eye protection / face protection.

P301 + P310 If swallowed: Immediately call a Poison Center and doctor.

P308 +P313 If exposed or concerned: Get medical advice / attention.

P331  DO NOT INDUCE VOMITING

P305 + P351 +P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

P306 + P360 IF ON CLOTHING: Rinse immediately contaminated clothing and skin with plenty of water before removing clothes.

P333 + P313 If skin irritation or rash occurs: Get medical advice / attention.

P337 + P313 If eye irritation persist: Get medical advice / attention.

P342 + P311 If experiencing respiratory symptoms: Call a POISON CENTR / DOCTOR

P361 + P364 Take off immediately all contaminated clothing and wash it before reuse.

_________________________________JBI, Inc._______

Chemical Blending

Safety Data Sheet								    Page 2

Yellow Monster

Signs and symptoms of exposure:

Eye:	Will cause serious eye irritation. Permanent damage may result if not flushed immediately.

Skin:	Can cause severe irritation and de-fatting of skin.

Ingestion: 	May  cause damage to throat, digestive system, and lungs.

Inhalation: Not a likely route of exposure, but if over-inhalation occurs symptoms include throat and respiratory irritation.

Medical Conditions Aggravated by Exposure: Over exposure could cause severe respiratory distress.

HMS Rating:  	Health -3  Flammability -0   Reactivity - 0     Personal  Protection - C

0= Insignificant 	1 = Slight	 2=Moderate      3=High                 4=Extreme

Section 3: Composition / Information on Ingredients

Substances

Composition  Information - Main constituents

| Chemical Name | CAS Number | %Range | OSHA PEL | ACGHI TLV |
| --- | --- | --- | --- | --- |
| Sodium Hydroxide | 1310-73-2 | <12% | 2 mg/m3 | Not Available |

Section 4: First Aid Measures

General Advice: 	Consult a physician. Show this safety data sheet to the doctor in attendance. Move out of dangerous area. 			Provide general supportive measures and treat symptomatically. Keep victim warm. Keep victim under 			observation.  Symptoms may be delayed.

Eye Contact: 	Seek medical attention immediately. DO NOT DELAY: Immediately irrigate eye continuously by holding eyes      		open and washing with fresh running water. Remove contact lenses, if present and easy to do. Irrigate for at 			least 15 minutes. Get medical attention  immediately.

Skin Contact:	Remove contaminated shoes and clothing immediately and wash skin with soap and water. In case of eczema 			or other skin disorders: Seek medical attention and take along these instructions. Wash contaminated clothing 			before reuse.

Ingestion:		Do not induce vomiting. Seek medical attention immediately. Drink water and follow with an acidic fruit juice 		and / or diluted vinegar. If vomiting occurs, keep ead low so that stomach content doesn’t get into the lungs.

Inhalation:		Remove person to well ventilated area. If breathing problems persist seek medical attention immediately.

Symptoms:	Severe eye irritation. May include stinging, tearing, redness, swelling, and blurred vision. Skin irritation. May 			include redness and pain. May cause an allergic skin reaction. Dermatitis. Rash.

_________________________________JBI, Inc._______

Chemical Blending

Safety Data Sheet								   Page 3

Yellow Monster

Section 5: Fire-Fighting Measures

Suitable Extinguishing Media: 	Use water fog. Foam. Dry chemical powder. Carbon Dioxide (CO2).

Unsuitable Extinguishing Media: Do not use water jet as and extinguisher, as this will spread the fire.

Fire and Explosive Properties: 	Flash Point: NA  Flammable Limits: LEL / UEL: Not Applicable.

Specific Hazard:		During fire, gases hazardous to health may be formed.

Protective Equipment: 	Wear a self contained breathing apparatus and full protective clothing. and body protections.

Fire fighting equipment/

Instructions:		Move Containers from fire area if you can do so without risk.

General Fire Hazards:		No unusual fire or explosion hazards noted.

Section 6: Accidental Release Measures

Steps to be taken if Material is Spilled or Leaked:

Provide adequate, good ventilation. Prevent spreading by using absorbent materials. Keep unnecessary personnel away. Keep people away from and upwind of spill / leak. Wear appropriate protective clothing and equipment during clean up. Avoid breathing mist or vapor. Do not touch damaged containers or spilled material unless wearing  appropriate protective clothing. Local authorities should be advised if spill cannot be contained. For large spills stop the flow of material, if able to do so without risk. Dike the spilled material, where this is possible. Cover with plastic sheet to prevent spreading. Absorb in vermiculite, dry sand or earth and place into containers. Following product recovery, flush area with water. For small spills wipe up with absorbent material. Clean surface thoroughly to remove residual contamination. Never return spills to original containers for re-use. Fore waste disposal, see section 13 of SDS. Avoid discharge into drains, water coursed or onto the ground.

Section 7: Handling and Storage

Handling:

Use only with adequate ventilation and Personal Protection Equipment (PPE). Rubber or plastic gloves and tight fitting goggles should be worn when handling product. Avoid all personal contact. Avoid contact wit eyes, skin, and clothing. Avoid prolonged exposure. Do not taste or swallow. When using, do not eat, drink, or smoke. Wash hands thoroughly after handling.

Storage:

Keep container tightly closed. Store in cool, dry, well-ventilated area. Store only in approved UN containers. Store in original tightly closed containers. Containers must be clearly labeled. Areas of use and storage should be ventilated adequately to reduce vapors below odor level.  Protect containers against physical damage and check regularly for leaks. Keep away from children and unauthorized personnel! Store away from incompatible materials such as aluminum, aluminum alloys, and metals.

________________________________JBI, Inc._______

Chemical Blending

Safety Data Sheet								Page 4

Yellow Monster

Section 8: Exposure Controls and Personal Protection

Ventilation:  Use in a well ventilated area.

Respirator Protection:  If required, a properly fitted NIOSH/MSHA approved respirator rated for the hazardous ingredients found in Section 3.

Skin Protection:  Chemical resistant, rubber or plastic gloves should be worn when handling product. Rubber boots. Ensure ready access to a safety shower. Cover all skin.

Eye Protection:    Wear completely enclosed tight fitting goggles and a full face shield. Contact lenses pose a special hazard: soft lenses may absorb irritants and all lenses concentrate them. Recommend no contact lenses.  Provide convenient eyewash stations.

General:  Keep away from food and drink. Observe good hygiene practices such as washing before eating, drinking , and/or smoking.

Routinely wash work clothing and protective equipment to remove contaminants. Contaminated clothing should not be allowed in the workplace.

Section 9: Physical and Chemical Properties

Boiling Point: 220’ Degrees F

Vapor Pressure (mm Hg @ 68’F): 0.00001hPa estimated

Vapor Density (AIR = 1): Not Available

Solubility in Water:  Complete

Specific Gravity (H2O = 1): 1.10

Melting Point: Not Available / Freezing Point: 32’F (0’C)

% Volatile:  Not determined

pH:  13

Appearance and Odor: Yellow with Butyl odor.

VOC Content: None

Section 10: Stability and Reactivity

Chemical Stability: Stable under normal use and temperature conditions.

Conditions to Avoid: Refer to Section 7. Avoid any contact with foodstuffs. Do not apply to or store in metal objects such as aluminum.  Avoid contact with metals including glass.

Hazardous Polymerization: Will not occur

Materials Incompatibility: Strong acids, oxidizing agents, metals including glass. Will etch glass. Corrosive to many metals.

Hazardous Decomposition Products: None known.

Section 11: Toxilogical Information

IARC, NTP, ACGIH, OSHA Carcinogen:  None

OSHA Regulated: Yes

Section 12: Ecological Information

This product is not classified as hazardous. However, this does not exclude the possibility that large or frequent spills can have a harmful or damaging effect on the environment.

________________________________JBI, Inc.________

Chemical Blending

Safety Data Sheet								Page 5

Yellow Monster

Section 13: Disposal Considerations

Collect and reclaim or dispose of  in sealed containers at licensed waste disposal site. Dispose of in accordance with local / state / national and international regulations. Empty containers may contain residue. Follow label warnings even after container is emptied. Empty containers should be taken to an approved waste handling site for recycling or disposal. The material and it’s container must be disposed of in a safe manner. Avoid sewers and drains.

Section 14: Transportation

D.O.T. REQUIREMENTS 49CFR 172.101: UN1824, SODIUM HYDROXIDE SOLUTION, 8, II, CORROSIVE

D.O.T. IATA, IMDG: Not regulated as dangerous goods.

Section 15: Regulatory Information

This product is a “Hazardous Chemical” as defined by the OSHA Hazard Comunication Standard, 29, CFR 1910.1200

US Federal Regulation

Chemical Name		CAS Number	%Range	OSHA PEL	ACGHI TLV

Sodium Hydroxide		1310-73-2		<12%	2mg/m3		NA

RQ (Reportable Quantity) 49CFR 172.101:

Component:		RQ (lbs)			For This Product

Sodium Hydroxide		100			Not Available

CAS 1310-73-2

TSCA (Toxic Substances Control Act) Status: The intentional ingredients of this product are listed.

CERCLA RQ-40CFR 302.4

Component		RQ (lbs)  		 For This Product

Sodium Hydroxide		100		 Listed

CAS 1310-73-2

SARA 302 COMPONENTS -40CFR 355 Appendix A:   Not regulated

SECTION 311/312 HAZARD CLASS - 40CFR 370.2

Immediate	   		(x)

Delayed 	                 		 (x)

Fire			( )

Reactive			(x )

Sudden Release of  Pressure	( )

WORKPLACE HAZARDOUS INFORMATION SYSTEM (WHMIS)

Ingredient				Max% by Wt.	LD50		LC50

Sodium Hydroxide 1310-73-2		<12%		Not Established          Not Established

California  Proposition 65:  None

________________________________JBI, Inc._______

Chemical Blending

Safety Data Sheet								Page 6

Yellow Monster

Section 16: Other Information

Although the information contained herein is believed to be reliable, it is furnished without warranty of any kind. This information is  not intended to be all-inclusive as to the manner and conditions of use, handling, and storage.

JBI urges each recipient of this Safety Data Sheet to study it carefully to become aware of and understand the hazards associated with the product.  Each recipient should inform all employees, customers, and all users of this product with information contained in this SDS.

JBI cannot anticipate all conditions under which this information and its product, or the products of other manufacturers in combination with its product may be used. It is the user’s responsibility to ensure safe conditions for handling, storage, and disposal of the product and to assume liability for loss, injury, damage, or expense due to improper use. The information relates only  to the specific material designated and may not be valid for such material used in combination with any other materials or in any process, unless specified in the text.